# THE TOWN AND COUNTRY PLANNING (ENVIRONMENTAL IMPACT ASSESSMENT) REGULATIONS 2017 SCREENING MATRIX

| 1. CASE DETAILS |  |  |  |
| --- | --- | --- | --- |
| Case Reference | DMPA/2024/1572 | Brief description of the project / development | Outline application (matters of access to be considered now with matters of layout, scale, appearance and landscaping reserved for later consideration) for demolition of all buildings and structures and a proposed residential development of up to 75 dwellings together with public open space and associated works on Land North of Linton Road, Castle Gresley |
| Applicant | Nightingale Land |  |  |
| LPA | South Derbyshire District Council (SDDC) |  |  |
| 2. EIA DETAILS |  |  |  |
| Is the project Schedule 1 development according to Schedule 1 of the EIA Regulations? |  |  | No |
| If YES, which description of development (THEN GO TO Q4) |  |  |  |
| Is the project Schedule 2 development under the EIA Regulations? |  |  | Yes |
| If YES, under which description of development in Column 1 and Column 2? |  |  | 10. Infrastructure projects (b) Urban development projects |
| Is the development within, partly within, or near a ‘sensitive area’ as defined by Regulation 2 of the EIA Regulations? |  |  | No |
| If YES, which area? |  |  |  |
| Are the applicable thresholds/criteria in Column 2 exceeded/met? |  |  | Yes – exceeded |
| If yes, which applicable threshold/criteria? |  |  | (iii) the overall area of development exceeds 5 ha Development size is 5.21ha |
| 3. LPA/SOS SCREENING |  |  |  |
| Has the LPA or SoS issued a Screening Opinion (SO) or Screening Direction (SD)? (In the case of Enforcement appeals, has a Regulation 37 notice been issued) |  |  | No |
| If yes, is a copy of the SO/SD on the file? |  |  |  |
| If yes, is the SO/SD positive? |  |  |  |
| 4. ENVIRONMENTAL STATEMENT |  |  |  |
| Has the appellant supplied an ES for the current or previous (if reserved matters or conditions) application? |  |  | No |

WHEN COMPLETING THIS DOCUMENT IN RELATION TO AN ENFORCEMENT APPEAL, THE UNDERSIGNED OFFICER HAS HAD REGARD TO THE PROJECT AS ALLEGED IN THE RELEVANT ENFORCEMENT NOTICE WHEN REFERING TO THE PROJECT / DEVELOPMENT.

Page 1/8

| A Screening Criteria Question | B Response to the Screening Criteria Question in Column A (Yes/No and explanation of reasons) |  | C Is a Significant Effect Likely? (Yes/No and explanation of reasons (nb if the answer in Column B is ‘No’, Column C is not applicable)) |  |
| --- | --- | --- | --- | --- |
|  | Briefly explain reasons and, if applicable and/or known, include name of feature(s) and proximity to site(s) |  | Is a significant effect likely, having regard particularly to the magnitude and spatial extent (including population size affected), nature, intensity and complexity, probability, expected onset, duration, frequency and reversibility of the impact and the possibility to effectively reduce the impact? If the finding of no significant effect is reliant on specific features or measures of the project envisaged to avoid, or prevent what might otherwise have been, significant adverse effects on the environment these should be identified in bold. |  |
| 5. NATURAL RESOURCES |  |  |  |  |
| 5.1 Will construction, operation or decommissioning of the project involve actions which will cause physical changes in the topography of the area? | Yes | There would be changes to the site during the construction phase as earth would be excavated to enable construction of foundations, drainage and roads. | No | Construction of the dwellings will include some excavation but not of such a scale that would be regarded as significant. The design would work with the existing topography of the site. Landscaping of the site would take place which would help reduce the long term impact of any topographical changes. |
| 5.2 Will construction or operation of the project use natural resources above or below ground such as land, soil, water, materials/minerals or energy which are non-renewable or in short supply? | Yes | The construction and operational phases of the development will use natural resources including land, soil, water, building materials and energy. The site includes 71.2% of land categorised as Grade 2 and 24.9% of land categorised as Grade 3a which is included within the definition of best and most versatile (BMV) agricultural land. | No | The site area is limited to 5.21 hectares, much of the land within South Derbyshire is BMV as such it is considered the loss is not significant. The site size falls below the 20ha threshold to warrant a consultation with Natural England. The development would need to be designed to reduce any likely significant effects on natural resource consumption and include sustainable building methods where feasible to minimise energy consumption. The natural resources required are commensurate with a project of this scale and nature and, as such, significant effects are unlikely. |
| 5.3 Are there any areas on/around the location which contain important, high quality or scarce resources which could be affected by the project, e.g. forestry, agriculture, water/coastal, fisheries, minerals? | Yes | The site includes 71.2% of land categorised as Grade 2 and 24.9% of land categorised as Grade 3a which is included within the definition of best and most versatile (BMV) agricultural land. There are no European designations applicable to the site. The site is located within a Minerals Safeguarding Area for Sand and Gravel. There are no coasts, fisheries, or tourism that are likely to be significantly affected by the development. | No | Due to the size of the development, it is considered that the loss of agricultural land would not be significant. Much of the agricultural land within South Derbyshire District is BMV grades 2 and 3 so it is considered the loss of 5.21 hectares is not significant. Furthermore, much of South Derbyshire District is within a Sand and Gravel Mineral Safeguarding Area so the loss of 5.21 hectares is not significant. |
| 6. WASTE |  |  |  |  |
| 6.1 Will the project produce solid wastes during construction or operation or decommissioning? | Yes | The proposed development is expected to result in waste materials from the preparation and undertaking of the development. Waste would also be generated by the operational phase of the proposed development. | No | Waste storage and disposal will be mitigated by existing legislative requirements so given the nature and scale of the development, no significant effects are likely. |

| A Screening Criteria Question | B Response to the Screening Criteria Question in Column A (Yes/No and explanation of reasons) |  | C Is a Significant Effect Likely? (Yes/No and explanation of reasons (nb if the answer in Column B is ‘No’, Column C is not applicable)) |  |
| --- | --- | --- | --- | --- |
| 7. POLLUTION AND NUISANCES |  |  |  |  |
| 7.1 Will the project release pollutants or any hazardous, toxic or noxious substances to air? | Yes | There would be emissions and dust associated with plant and vehicles during the construction phase. There would also be emissions associated with traffic movements during the operational phase of the proposed development. The proposed development is for residential use which is not associated with hazardous substances or toxic emissions to air. There is not anticipated to be a requirement to store large volumes of hazardous materials. Any such materials would be stored and handled in accordance with relevant legislation. | No | Dust generation would be managed in accordance with standard best practice measures, enforced through a Construction and Environmental Management Plan. Pollutant emissions associated with plant and non-road mobile machinery would be operated in accordance with the Construction and Environmental Management Plan to ensure pollutant emissions are reduced and avoided where possible. During operation of the development, a Travel Plan will need to be implemented to encourage use of sustainable transport. |
| 7.2 Will the project cause noise and vibration or release of light, heat, energy or electromagnetic radiation? | Yes | The potential exists for noise effects to result from the construction processes and operational activities associated with the proposed development. During construction, the potential exists for light pollution (at night) associated with construction activities. It is not anticipated that heat, energy or electromagnetic radiation will be caused or released. | No | Construction noise would be in the context of the site being bordered a main road to the north and the existing built development to the east and south and so is considered unlikely to cause significant disturbance. The nearest sensitive receptors include existing residential properties to the east and south. Effects will be managed in accordance with best practice measures implemented through a Construction and Environmental Management Plan. All external lighting and illumination would be required to be designed carefully and located sensitively. It is not anticipated that significant adverse effects will arise. |
| 7.3 Will the project lead to risks of contamination of land or water from releases of pollutants onto the ground or into surface waters, groundwater, coastal waters or the sea? | Yes | The site is known to include contaminated land ground gas which is considered to be of a low to moderate (localised) risk. Construction could mobilise silts and sediments and increase nutrient inputs into local watercourses. Site construction will also require the use of and storage of fuels and other chemicals and production materials on site during construction. Post construction, the use of private and commercial road vehicles could increase the risk of ground and waterbody contamination associated with increased traffic movements. | No | The land use proposed is not highly contaminative. Surface water run-off and foul water drainage will be managed on-site during the construction and operational phases. A contaminated land assessment, Flood Risk Assessment and Drainage Strategy will be required to support any future planning application. Planning conditions would be used to mitigate any adverse effects/secure mitigation. The proposals are not expected to result in any significant adverse effects to land or water. |
| 7.4 Are there any areas on or around the location which are already subject to pollution or environmental damage, e.g. where existing legal environmental standards are exceeded, which could be affected by the project? | No | The site is not within or in proximity to any such areas. |  |  |
| 8. POPULATION AND HUMAN HEALTH |  |  |  |  |
| 8.1 Will there be any risk of major accidents (including those caused by climate change, in accordance with scientific knowledge) during construction, operation or decommissioning? | No | The site is wholly within Flood Zone 1 and there are no known risks associated with the development of the site. |  |  |
| 8.2 Will the project present a risk to the population (having regard to population density) and their human health during construction, operation or decommissioning? (for example due to water contamination or air pollution) | Yes | During the construction phase, dust and pollutant emissions are likely to be generated by the operation of plant and vehicles. Other emissions to air, for example from machinery and road vehicles, as well as the mobilisation of pollutants to ground are also possible given the nature of the scheme. | No | Dust and pollutants during the construction phase would be managed in accordance with standard best practice measures, enforced through the Construction and Environmental Management Plan, and are not anticipated to generate significant adverse effects to human health. Remediation (and certification that this has been undertaken) for any contamination on site would be a requirement of any planning permission. |

| A Screening Criteria Question | B Response to the Screening Criteria Question in Column A (Yes/No and explanation of reasons) |  | C Is a Significant Effect Likely? (Yes/No and explanation of reasons (nb if the answer in Column B is ‘No’, Column C is not applicable)) |  |
| --- | --- | --- | --- | --- |
| 9. WATER RESOURCES |  |  |  |  |
| 9.1 Are there any water resources including surface waters, e.g. rivers, lakes/ponds, coastal or underground waters on or around the location which could be affected by the project, particularly in terms of their volume and flood risk? | Yes | The site is located away from main rivers and significant water resources. The site is wholly within Flood Zone 1 so it is at low risk of flooding from rivers. | No | A Flood Risk Assessment/Drainage Strategy will be required in support of any planning application. Surface water run-off and foul water drainage will be managed on site during the construction and operational phases. A scheme incorporating sustainable drainage measures will be required for the development. |
| 10. BIODIVERSITY (SPECIES AND HABITATS) |  |  |  |  |
| 10.1 Are there any protected areas which are designated or classified for their terrestrial, avian and marine ecological value, or any non-designated / non-classified areas which are important or sensitive for reasons of their terrestrial, avian and marine ecological value, located on or around the location and which could be affected by the project? (e.g. wetlands, watercourses or other water-bodies, the coastal zone, mountains, forests or woodlands, undesignated nature reserves or parks. (Where designated indicate level of designation (international, national, regional or local))). | Yes | There is a Local Wildlife Site immediately adjoining the northern boundary of the site (White Lady Springs) and the site is a candidate Local Wildlife Site (High Cross Bank Field). There is no other known nearby protected sites. | No | The site is agricultural land with the trees on the site perimeter. A Preliminary Ecological Assessment and other appropriate or identified species and habitat surveys would be required as part of the planning application. Mitigation and enhancement measures (including Biodiversity Net Gain) would be subject to control using planning conditions/obligations. |
| 10.2 Could any protected, important or sensitive species of flora or fauna which use areas on or around the site, e.g. for breeding, nesting, foraging, resting, over-wintering, or migration, be affected by the project? | Yes | The site is arable land with some mature trees, hedges and vegetation around the perimeter. | No | It is likely that any effects that may impact species can be managed via survey work and the imposition of planning conditions to secure appropriate methods of working and mitigation measures if necessary. |

| A Screening Criteria Question | B Response to the Screening Criteria Question in Column A (Yes/No and explanation of reasons) |  | C Is a Significant Effect Likely? (Yes/No and explanation of reasons (nb if the answer in Column B is ‘No’, Column C is not applicable)) |  |
| --- | --- | --- | --- | --- |
| 11. LANDSCAPE AND VISUAL |  |  |  |  |
| 11.1 Are there any areas or features on or around the location which are protected for their landscape and scenic value, and/or any non-designated / non-classified areas or features of high landscape or scenic value on or around the location which could be affected by the project?1 Where designated indicate level of designation (international, national, regional or local). | No | The site is not located in an area designated for landscape or scenic value. | No | Significant landscape and visual amenity effects are not likely due to the nature of the development proposal (residential) which is of a relatively limited scale and compatible with other land uses and residential development in the immediate vicinity. This will also be appropriately mitigated using planning conditions such as the provision of a landscaping scheme for the development. |
| 11.2 Is the project in a location where it is likely to be highly visible to many people? (If so, from where, what direction, and what distance?) | Yes | The development is likely to be most visible from Hillside Road, as well as from Bridleway no. 3. | No | Significant landscape and visual amenity effects are not likely due to the nature of the development proposal (residential) and the areas of public open space and landscaping proposed. Whilst visible from the Bridleway, it would still be at some distance and would be seen in the context of existing development. This will also be appropriately mitigated using planning conditions such as the provision of a landscaping scheme for the development. |
| 12. CULTURAL HERITAGE/ARCHAEOLOGY |  |  |  |  |
| 12.1 Are there any areas or features which are protected for their cultural heritage or archaeological value, or any non-designated / classified areas and/or features of cultural heritage or archaeological importance on or around the location which could be affected by the project (including potential impacts on setting, and views to, from and within)? Where designated indicate level of designation (international, national, regional or local). | Yes | The site does not accommodate any heritage assets. To the north of the site is Castle Gresley Motte and Bailey Castle Scheduled Ancient Monument. The area is also considered to have archaeological potential. | No | The built area of development has been proposed further away from the Scheduled Ancient Monument so as not to detrimentally impact on its setting. Archaeological potential can be addressed through appropriate planning conditions. No significant impacts are expected on the basis of the proposed layout. |

1  See question 8.1 for consideration of impacts on heritage designations and receptors, including on views to, within and from designated areas.

| A Screening Criteria Question | B Response to the Screening Criteria Question in Column A (Yes/No and explanation of reasons) |  | C Is a Significant Effect Likely? (Yes/No and explanation of reasons (nb if the answer in Column B is ‘No’, Column C is not applicable)) |  |
| --- | --- | --- | --- | --- |
| 13. TRANSPORT AND ACCESS |  |  |  |  |
| 13.1 Are there any routes on or around the location which are used by the public for access to recreation or other facilities, which could be affected by the project? | Yes | Castle Gresley PROW no.5 runs to the north of the site and PROW/Bridleway no. 3 runs to the west west of the site. | No | The development can be designed to ensure that the Public Right of Way is not affected, as such it is not envisaged that significant effects will arise. The development is proposed to link up to Bridleway no.5 |
| 13.2 Are there any transport routes on or around the location which are susceptible to congestion or which cause environmental problems, which could be affected by the project? | No | There are no known environmental issues associated with nearby transport routes, although local knowledge would suggest that the A444 (Burton Road) is a busy thoroughfare. | No | The road congestion is not severe so it is unlikely that the number of dwellings proposed would cause significant effects to arise. Any planning application would need to include an assessment of the operation of this junction to establish if any mitigation measures are required to enable it to satisfactorily accommodate the traffic generated by the proposed development. |
| 14. LAND USE |  |  |  |  |
| 14.1 Are there existing land uses or community facilities on or around the location which could be affected by the project? E.g. housing, densely populated areas, industry / commerce, farm/agricultural holdings, forestry, tourism, mining, quarrying, facilities relating to health, education, places of worship, leisure /sports / recreation. | Yes | There is housing to the east and south of the site. Within Castle Gresley there are community facilities e.g. Shops on Linton Road, Mount Pleasant Recreation Ground (300m). Additionally within the urban area of Swadlincote there is considered to be a wider range of community facilities to meet the needs of those within the area. | No | It is considered the proposed development can be designed so as to avoid significant effects on the neighbouring uses. The loss of 2.93ha of agricultural land in the wider context of the District is not likely to be significant. It is considered existing community facilities will not be significantly affected and relevant planning contributions will be sought where there is a need/shortfall in provision. |
| 14.2 Are there any plans for future land uses on or around the location which could be affected by the project? | No | There are no allocations or recently approved applications immediately adjacent to or on the site. No significant effects are considered to result. |  |  |
| 15. LAND STABILITY AND CLIMATE |  |  |  |  |
| 15.1 Is the location susceptible to earthquakes, subsidence, landslides, erosion, or extreme /adverse climatic conditions, e.g. temperature inversions, fogs, severe winds, which could cause the project to present environmental problems? | No | The location is not identified as one with land stability or climate implications that would affect the proposed development. |  |  |

| A Screening Criteria Question | B Response to the Screening Criteria Question in Column A (Yes/No and explanation of reasons) |  | C Is a Significant Effect Likely? (Yes/No and explanation of reasons (nb if the answer in Column B is ‘No’, Column C is not applicable)) |  |
| --- | --- | --- | --- | --- |
| 16. CUMULATIVE EFFECTS |  |  |  |  |
| 16.1 Could this project together with existing and/or approved development result in cumulation of impacts together during the construction/operation phase? | Yes | This site has the potential to add to further allocated/committed residential development within South Derbyshire. | No | There are no significant allocated residential sites in the immediate locality so significant cumulative effects are unlikely. |
| 17. TRANSBOUNDARY EFFECTS |  |  |  |  |
| 17.1 Is the project likely to lead to transboundary effects?2 | No | It is considered that there would be no transboundary impacts associated with the proposed development given the scale of housing proposed. |  |  |

2	The Regulations require consideration of the transboundary nature of the impact. Due to the England’s geographical location the vast majority of TCPA cases are unlikely to result in transboundary impacts.

| 18. CONCLUSIONS – ACCORDING TO EIA REGULATIONS SCHEDULE 3 |  |  |
| --- | --- | --- |
| It is considered the characteristics of the development in terms of: - the size and design of the development (75 dwellings); - cumulation with other existing and/or approved development (none identified); - use of natural resources (limited by scale of development); - production of waste (limited by scale of development); - pollution and nuisances (limited increase in noise disturbance during construction and operational phases is likely); - risk of major accidents (none identified) and - the risks to human health (no significant detrimental implications on nearby receptors) will not result in significant effects. The site is not located in a sensitive location in terms of land use designation or the capacity of natural resources/environment. The site is not located within any ‘sensitive areas’ as defined by the EIA Regulations 2017, and with appropriate mitigation, either embedded into the development design or secured through planning condition, will not give rise to likely significant effects. The types and characteristics of the potential impact of the development will be limited by the scale of the site and nature of the development proposed. No significant transboundary or cumulative impact is expected. The scheme is not considered to be EIA development. |  |  |
| 19. SCREENING DECISION |  |  |
| If a SO/SD has been provided do you agree with it? |  |  |
| Is it necessary to issue a SD? |  |  |
| Is an ES required? | No |  |
| 20. ASSESSMENT (EIA REGS SCHEDULE 2 DEVELOPMENT) | OUTCOME |  |
| Is likely to have significant effects on the environment | ES required |  |
| Not likely to have significant effects on the environment | ES not required |  |
| More information is required to inform direction | Request further info |  |
| 21. REASON FOR SCREENING |  |  |
| The proposed project is screened, in accordance with the relevant legislation, under Schedule 2 Section 10 (b) as it is an urban development project for up to 75 dwellings and the area of the proposed development exceeds 5 hectares. |  |  |

NAME Jessica Clarke-Roome Approved By: Sarah Beeby

DATE 04/03/2026

Page 8/8